The standard controller is replaced by a fully integrated microprocessor that monitors and controls the chiller section and refrigeration system.  The microprocessor shall control the chiller section to prevent freeze-up.   Additionally, the microprocessor shall incorporate the following features:

- Chiller Temperature Alarm

- Condensate Drain Time Adjustment

- % Savings

- Refrigerant Compressor Running Hours

- Dryer Running Hours

- Auto Restart

- Remote Start/Stop

- Crank Case Heater Delay

- RS232/485-Remote Communication – Ready

- High Condensate Level Alarm – Ready

- Advanced Diagnostic memory

- Battery Back-up Memory

- Refrigerant Discharge pressure

- Refrigerant Suction Pressure

- Refrigerant Suction Temperature

- Air Inlet Pressure

- Outlet Air Pressure

- Air Inlet Air Temperature

- Air Outlet Temperature

Microprocessor shall also incorporate field programmable dew point settings to allow the dryer to be more closely matched to seasonal demands.  A higher dew point setting shall allow the refrigerant compressors to experience a lighter load thereby conserving more energy and further reducing compressor wear and tear.